# Rakshak Project Handout

A local-first satellite image review demo for a college hackathon

Rakshak helps a user review overhead images and vessel-related data in one dashboard. It currently has two separate workflows: YOLO vessel candidates on xView-style imagery, and Sentinel-2 scene context with PipeV4 locations and AIS review. All results are prompts for human review, not confirmed ships or threats.

| 2 modes One switchable dashboard | Local-first API and review data stay on this machine | Human reviewed Candidates are not threat conclusions |
| --- | --- | --- |

# What each mode does

| MODE 1 COMPUTER VISION | MODE 2 OPERATIONAL SURVEILLANCE |
| --- | --- |
| xView / xView3-style imagery Runs the local YOLO vessel detector. Candidate boxes are drawn over the uploaded image with confidence labels. A 40% minimum confidence setting hides weaker candidates; lower it and reanalyze to see weaker ones. | Sentinel-2 GeoTIFF imagery Displays the scene and its map footprint. If a matching PipeV4 month is available, provider locations are filtered to the scene bounds and projected as purple markers. Analysts can review AIS association, view a heatmap, and create a review sequence. |

# Current working features

Upload PNG, JPG, TIFF, or GeoTIFF images through the local dashboard.

View model boxes, confidence scores, and a review list in Computer Vision mode.

View Sentinel-2 scene context, provider detections, and local AIS review controls in Operational Surveillance mode.

Explore maps and heatmaps; save a review-only mission sequence.

Run the React/TypeScript frontend with a Python FastAPI backend and local SQLite storage.

Demo address  http://127.0.0.1:5173/

# Data, accuracy and next steps

The current build is a hackathon prototype. Its most useful next improvement is better labeled imagery that matches the images people will actually upload.

# Technology and data in the current build

Frontend: React, TypeScript, Vite, Leaflet maps, and Recharts.

Backend: Python FastAPI, SQLite, rasterio/GDAL for image previews and georeferencing, and Ultralytics YOLO for Computer Vision mode.

Local data: xView-derived vessel tiles for training; Sentinel-2 images and monthly PipeV4 provider CSVs for scene context.

Deployment: runs on the local computer. OpenStreetMap basemap tiles require an internet connection.

# What the current results mean

| 0.277 xView validation mAP@50 | 897 / 183 training / validation image tiles | 40% default confidence display threshold |
| --- | --- | --- |

The model is still prone to false alarms and missed vessels. The reported metric is from xView validation only; it does not measure accuracy on Sentinel-2 or Indian coastal images. Raising the confidence threshold reduces weak detections, but can hide real ships too.

# Why Sentinel-2 needs a different detector

Sentinel-2 visible imagery is 10 metres per pixel, while xView imagery is about 0.3 metres per pixel. A small vessel may cover very few Sentinel-2 pixels, so an xView-trained model cannot reliably outline its hull there. Operational Surveillance markers are PipeV4 coordinates projected onto the image; they are not image-derived ship outlines.

# Resources that would improve accuracy

False-alarm examples from your images, with each incorrect box marked as “not a ship.” Include docks, wakes, clouds, rocks, and bright water if they caused errors.

More images from the same sensor, resolution, coast, season, and conditions as your demo. Label real ships with bounding boxes; include clear negative scenes with no ships.

For a first fine-tuning experiment, aim for roughly 200–500 representative annotated image tiles, then grow the set as errors are found. This is a practical starting target, not a guarantee.

For Sentinel-1 SAR / dark-vessel detection, use a SAR-specific dataset and model such as xView3 rather than mixing SAR and optical images into this checkpoint.

# Good next demo workflow

1. Choose Computer Vision for xView-style overhead images, upload, and adjust the confidence control.  2. Choose Operational Surveillance for Sentinel-2 GeoTIFFs and inspect only the available date-and-footprint-matched PipeV4 records.  3. Review each candidate manually and record false alarms and missed ships for the next training round.

Reference links  ESA Sentinel-2 resolution   ·   xView benchmark   ·   xView3 vessel dataset

Scope note  This is a classroom demonstration, not an operational surveillance or threat-classification system. Do not use its outputs as the sole basis for real-world decisions.